qlaser_dacs_pulse

32-Channel Pulse Generator — Description and Register Map

Sources: qlaser_dacs_pulse_ch1.vhdl  (wrapper),  qlaser_dacs_pulse_channel_ramp.vhdl  (channel).

# Overview

These two files form a parent/child pair in the qlaser DAC pulse-output path. The wrapper presents a single register/AXI-Stream interface to the CPU and fans it out to 32 identical channel slots; each slot is one instance of the channel entity, which drives one 16-bit AXI-Stream toward a JESD DAC lane.

| Entity / Architecture | File | Role |
| --- | --- | --- |
| qlaser_dacs_pulse | _ch1.vhdl | 32-channel top level: sequence state machine, channel-select addressing, status/error aggregation |
| qlaser_dacs_pulse_channel | _channel_ramp.vhdl | One channel; the ramp architecture is a JESD-debug sawtooth generator |

The wrapper instantiates architecture ramp for the first G_NCHANS channels and architecture empty for the remainder. Other channel architectures (e.g. wave_4K_pdef_1Kx32) exist and are selected by swapping the instantiated architecture. This document covers the ch1 wrapper and the ramp channel.

# Part A — qlaser_dacs_pulse (ch1)

The top-level wrapper. A single shared 24-bit sequence counter and state machine drive all channels in lock-step. To keep the CPU address bus narrow, channels are not fully address-mapped; instead a 32-bit channel-select register chooses which channel block(s) a channel-space access targets. Writes may be broadcast to many channels at once (e.g. clear all channel RAMs); reads must select exactly one channel.

## Generic

| Generic | Default | Description |
| --- | --- | --- |
| G_NCHANS | 1 | Number of 'full' channels. Channels 0..G_NCHANS-1 are instantiated as full (ramp) blocks; G_NCHANS..31 are empty blocks that read back 0xCCCCCCCC when selected. |

## Interfaces

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| enable | in | 1 | DAC interface is running; rising edge advances the SM out of reset |
| trigger | in | 1 | Rising edge starts a pulse-generation sequence on enabled channels |
| jesd_syncs | in | 32 | Per-DAC JESD sync status from the FMC216 board |
| ready | out | 1 | All enabled JESD channels are synced (= not error) |
| busy | out | 1 | A sequence is running (OR of channel-busy and SM-busy) |
| error | out | 1 | Instantaneous JESD sync error on any enabled channel |
| error_latched | out | 1 | JESD lost sync after ready; sticky, cleared by trigger/reset |
| cpu_addr | in | 13 | Word address (bit 12 selects register vs channel space) |
| cpu_wdata / cpu_rdata | in/out | 32 | Write / read data |
| cpu_wr / cpu_sel / cpu_rdata_dv | — | 1 | Write enable / block select / read-data-valid |
| axis_treadys | in | 32 | Per-channel downstream ready |
| axis_tdatas | out | 32×16 | Per-channel stream data (t_arr_slv32x16b) |
| axis_tvalids / axis_tlasts | out | 32 | Per-channel valid / last |

## Address decode

The 13-bit cpu_addr is split by its top bit:

| cpu_addr[12] | Region | Decode |
| --- | --- | --- |
| 1 | Local register block | Registers decoded on cpu_addr[3:0] (see map below). Upper bits are don't-care. |
| 0 | Channel space | cpu_addr[11:0] forwarded to channel blocks; REG_CH_SELS picks which channel(s) respond. |

Channel-select mechanism: to write, set the target bits in REG_CH_SELS then write to channel-space addresses — data broadcasts to every selected channel. To read, set exactly one bit; channel read data is OR-combined, so multiple selected channels merge into garbage.

## Register map (local register block, cpu_addr[12] = 1)

| Offset | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x0 | REG_SEQUENCE_LEN | R/W | 0x800000 | Max sequence length, 24-bit, in clk cycles. S_RUN counts up to this value. |
| 0x1 | REG_CH_SELS | R/W | 0xFFFFFFFF | Channel-select mask for CPU access to channel space (1 bit per channel). |
| 0x2 | REG_CH_EN | R/W | — (not reset) | Per-channel run enable; also the mask for JESD-sync error detection. |
| 0x3 | REG_STATUS | R | — | Aggregate busy/error status (see bit table). |
| 0x4 | REG_STATUS_JESD | R | — | Per-channel latched JESD loss-of-sync (errs_jesd_latched). |
| 0x5 | REG_CNT_TIME | R | — | Current sequence time counter, 24-bit (sm_cnt_time). |
| others | — | R | 0x00000000 | Reads as zero. Writes ignored. |

## REG_STATUS (0x3) bit fields

| Bit | Name | Meaning |
| --- | --- | --- |
| 0 | busy | A channel or the state machine is active (busy_i). |
| 4 | error | Any enabled channel currently has JESD sync de-asserted (any_errs_jesd). |
| 31:5, 3:1 | — | Reserved, read 0. |

## Sequence state machine

| State | Behaviour |
| --- | --- |
| S_RESET | Power-up state. Waits for a rising edge on enable, then moves to S_IDLE. Holds counters and latched errors cleared. |
| S_IDLE | Waits for a rising edge on trigger, then moves to S_RUN, asserts SM-busy, and clears latched JESD errors. |
| S_RUN | Increments sm_cnt_time each clock until it reaches REG_SEQUENCE_LEN, then returns to S_IDLE. Latches per-channel JESD errors while running. |

## Status / error logic

- ch_errs_jesd[I] = REG_CH_EN[I] AND NOT jesd_syncs[I] — an enabled channel whose JESD sync is low.

- error = any_errs_jesd  (OR of the above);  ready = NOT any_errs_jesd.

- error_latched is the OR of the sticky per-channel latch, set during S_RUN, cleared by reset/trigger.

- busy = OR(ch_busy) OR sm_busy.

## Implementation notes

REG_CH_EN (0x2) is not initialised in the reset branch. It powers up in an undefined state until written, yet it gates every channel enable and masks JESD error detection. Firmware should write REG_CH_EN explicitly before relying on enable or status behaviour.

- Channel read data is OR-combined across all 32 slots (plus the register block). Only one bit may be set in REG_CH_SELS for a read, or channel contents merge. This is by design — the trade-off for the narrow address bus.

- In S_RUN the sm_cnt_time = 0xFFFFFF check and its else branch both go to S_IDLE, so that explicit max-count test is redundant; the sequence ends when sm_cnt_time reaches REG_SEQUENCE_LEN.

- The shared sm_cnt_time is passed to every channel as cnt_time, but the ramp channel architecture ignores it (see Part B).

# Part B — qlaser_dacs_pulse_channel (ramp)

A single pulse-output channel, JESD-debug variant. When enabled it emits a free-running 16-bit sawtooth: an accumulator advances by a programmable step each clock and wraps back to zero once it would exceed 0xFFFF. With a step of 1 this sweeps the full 0..0xFFFF range; larger steps sweep faster / coarser.

## Interfaces

| Signal | Dir | Width | Function |
| --- | --- | --- | --- |
| enable | in | 1 | Run the channel. While low, output holds 0 and tvalid is low |
| start | in | 1 | Sequence-start (unused in ramp — see notes) |
| cnt_time | in | 24 | Time since trigger (unused in ramp — see notes) |
| busy | out | 1 | Mirrors enable (registered, 1-cycle delay) |
| err_wave | out | 1 | Tied to 0 — no error condition in this architecture |
| cpu_addr | in | 12 | Word address |
| cpu_wdata / cpu_rdata | in/out | 32 | Write / read data |
| cpu_wr / cpu_sel / cpu_rdata_dv | — | 1 | Write enable / block select / read-data-valid |
| axis_tready | in | 1 | Downstream ready (unused — see notes) |
| axis_tdata | out | 16 | Ramp value, low 16 bits of the accumulator |
| axis_tvalid | out | 1 | High while enabled |
| axis_tlast | out | 1 | Tied to 0 |

## Behaviour

- busy <= enable  (registered).

- While enable = 1: accumulator cnt_ramp += RAMP_STEP each clock; if the next value would exceed 0xFFFF it wraps to 0. axis_tvalid = 1.

- While enable = 0: accumulator held at 0, axis_tvalid = 0.

- axis_tdata = cnt_ramp[15:0];  axis_tlast = 0 always.

## Register map

| Word Addr | Name | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x000 | RAMP_STEP | R/W | 0x0001 | Ramp step size in bits [15:0]; added to the accumulator each clock. |
| 0x001 | ID / VERSION | R | 0x00001234 | Fixed read-only marker constant. |
| others | — | R | 0xCCCCCCCC | Unmapped — returns debug marker. Writes ignored. |

On read, RAMP_STEP returns 0x0000 & step[15:0]. Only bits [15:0] of a write to 0x000 are used.

## Implementation notes

- Three ports are unused in this architecture: start, cnt_time, and axis_tready. The ramp free-runs on enable alone and does not honour AXI-Stream backpressure. That is fine as a debug source into an always-ready JESD path, but it is not a general-purpose stream master.

- err_wave is tied to 0 and busy simply follows enable — this channel reports no error and no meaningful busy handshake.

- The wrapper instantiates this (ramp) architecture for full channels and empty for the rest; the 0x001 marker here (0x1234) differs from the ID scheme used in other qlaser blocks.