# Spec axi_if_cpubus.

This is a Vivado IP block that is used in a Vivado block diagram.  The block is connected as an AXI-lite bus slave and allows the block diagram CPU to have a simplified bus interface to user HDL blocks outside of the block diagram. It does not support burst transfers.

The simplified CPU bus interface has an output 18-bit (byte) address bus, 32-bit write data, with read and write strobes. There is a 32-bit input data bus with a data valid signal for data returned by read operations.

The user HDL block does not need to provide any response to a bus write transaction.

For a read transaction the user HDL block should drive ‘cpu_rdata_dv’ high when it is driving valid output data on the ‘cpu_rdata’ bus. Once ‘cpu_rd’ is set low then the ‘cpu_rdata_dv’ input must be driven low. The block passes through the AXI clock ‘s00_axi_aclk’ to the ‘clk_cpu’.

# Vivado Symbol

# Block I/O

-- AXI-lite slave interface

s00_axi_aresetn         Active low reset

s00_axi_aclk            AXI-lite bus clock

s00_axi                 AXI-lite bus

-- CPU Control interface connected to block diagram pins

cpu_clk                 : out std_logic;

cpu_wr                  : out std_logic;

cpu_rd                  : out std_logic;

cpu_addr                : out std_logic_vector(17 downto 0);

cpu_wdata               : out std_logic_vector(31 downto 0);

cpu_rdata               : in  std_logic_vector(31 downto 0);

cpu_rdata_dv            : in  std_logic;

# Register table.

The block has no internal registers but is assigned a base address and has a byte address range of 0x3FFFF.  Writes and reads within this space are passed through to the CPU bus.

# Document Version

1.0 	Initial version